# BOQ text extraction - 1290886

Source PDF: boq.pdf

## Page 1

Lot Detail

Lot No. :

Ban-61/2025-26

Lot Description :

Construction of Regulator (3-Vent: 1.50m x 1.80m) on Kumuria Khal at Sonali Bazar, Union- Mognama, Upazilla-Pekua,

Dist- Coxs Bazar under "Operational Expenditure for Goods and Services (Economical code-3258137)" under Cox's

Bazar W D Division-2, BWDB, Bandarban during FY 2025-26.

Bill of Quantities

Table

Name :

Bill of Quantities

Item

no.

Group

Item

Code (if

any)

Description of Item

Measurement

Unit

Quantity

Unit Price

In figures (BDT)

Unit

Price

In

Words

(BDT)

Total

Price

In

Figures

(BDT)

Total

Price

In

Words

(BDT)

1

1

04-180-

00

(BWDB)

Site preparation by manually

removing all miscellaneous

objectional materials from

entire site and removing soil up

to 15cm depth including

uprooting stumps, jungle

clearing, leveling dressing etc.

complete as per direction of

Engineer in charge.

Sqm

18000

2

1

01.1

(PWD)

Erection and maintenance of

semi permanent site office and

removal of the same after

completion of work in

accordance with the conditions

of contract. In addition to the

office required for own use, the

contractor shall provide and

maintain furnished site office

for the use of Engineer-in-

charge and his staff. The site

office must have tiles floor,

adequate foundation, brick

walls, false ceiling of gypsum

board and all windows are to

be glazed, shuttered and

provided with steel grill.

Outside and inside wall surface

are to be painted on plaster

acceptable to the Engineer-in-

charge. The site office shall be

maintained in a secure

condition by the contractor until

the completion of the contract

and shall be provided with

electricity, water supply, wash

rooms and sewerage facilities.

All doors shall be fitted with

approved locks and windows

shall be provided with

screen/blinds. Before

construction the contractor

shall submit plans and drawings

showing proposed details and

location for the site ofice,

including foundations, access

roads, shades, layout of

electrical and water supply and

hard standings there for the

approval of Engineer-in-charge.

The Engineer-in-charge may

require revision of the plans

prior to giving approval for

construction. The contractor

shall also submit detailed

proposed furniture, fittings and

other items of equipment and

plant to the Engineer-in-charge

job

1

## Page 2

for approval. These items shall

be of the standard quality

suitable for site. The office,

complete with furnishings,

fittings, access roads and hard

standings shall be ready for

occupation by the Engineer-in-

charge within 28 days of the

date when the contractor first

occupies the site. The

contractor shall provide day

and night guards and an

attendant for the field office. At

the end of the contract all

materials, equipment and plant,

furniture, fittings recovered

from dismantling the office and

removing access road will be

the property of the contractor.

No interim payment shall be

certified unless engineer's

office with required facilities

are constructed and accepted

by the Engineer-incharge. (

This is a time related item;

proportionate payment for this

item shall be made distributing

in each bill on the basis of

percentage progress of the

whole works under contract)

Engineer's site office of

minimum 38 sqm plinth area

with providing necessary

facilities including office

furniture, 24000 BTU air

cooler, umbrella, crookeries,

water purifier, PC with monitor,

uninterruptible power supply

(UPS), laser jet printer

(minimum 20 ppm), LED flood

light fittings (Halogen) for sight

security etc. 01.1.3: Engineer's

site office of minimum 38 sqm

plinth area with providing

necessary facilities including

office furniture, 24000 BTU air

cooler, umbrella, crookeries,

water purifier, PC with monitor,

uninterruptible power supply

(UPS), laserjet printer

(minimum 20 ppm), LED flood

light fittings(Halogen) for sight

security etc.

3

1

40-620-

00

(BWDB)

Supplying new Synthetic bags

at site :

40-620-20: Capacity

75 kg.

No

24375

4

1

40-640-

00

(BWDB)

Labour charge for filling empty

gunny or synthetic bags with

sand or earth, sewing the end

with sutly, including carring and

placing under water position

within the site as per direction

of Engineer -in-Charge. 40-

640-20: Capacity : 75 kg (Fill

Volume = 0.040 cum)

No

24375

5

1

16-450-

00

(BWDB)

Earth work by manual labour

with clayey soil (minimum 30%

clay, 0-40% silt and 0-30%

sand) in construction of cross

bundh/ ring bundh as per

design and specification with all

leads and lifts, throwing the

earth in layers not exceeding

150mm in thickness, including

breaking clods, rough dressing,

clearing the jungle, removing

cum

5264.780

## Page 3

stumps, dug bailing and 75mm

cambering etc. complete as

per direction of Engineer in

charge.

6

1

02.1

(PWD)

Earth work in excavation in all

kinds of soil for foundation

trenches including layout,

providing center lines, local

bench-mark pillars, levelling,

ramming and preparing the

base, fixing bamboo spikes and

marking layout with chalk

powder, providing necessary

tools and plants, protecting and

maintaining the trench dry etc.,

stacking, cleaning the

excavated earth at a safe

distance out of the area

enclosed by the layout etc. all

complete and accepted by the

Engineer-in-charge, subject to

submit method statement of

carrying out excavation work to

the Engineer-in-charge for

approval.

However, engineer's

approval shall not relieve the

contractor of his responsibilities

and obligations under the

contract.

02.1.2: Earthwork in

excavation in foundation

trenches up to 1.5 m depth and

maximum 10 m lead

cum

8749.360

7

1

02.1

(PWD)

Earth work in excavation in all

kinds of soil for foundation

trenches including layout,

providing center lines, local

bench-mark pillars, levelling,

ramming and preparing the

base, fixing bamboo spikes and

marking layout with chalk

powder, providing necessary

tools and plants, protecting and

maintaining the trench dry etc.,

stacking, cleaning the

excavated earth at a safe

distance out of the area

enclosed by the layout etc. all

complete and accepted by the

Engineer-in-charge, subject to

submit method statement of

carrying out excavation work to

the Engineer-in-charge for

approval.

However, engineer's

approval shall not relieve the

contractor of his responsibilities

and obligations under the

contract.

02.1.3: Added rate

for each additional 0.5 meter

depth exceeding 1.5 meter.(7)

cum

19948.870

8

1

02.1

(PWD)

Earth work in excavation in all

kinds of soil for foundation

trenches including layout,

providing center lines, local

bench-mark pillars, levelling,

ramming and preparing the

base, fixing bamboo spikes and

marking layout with chalk

powder, providing necessary

tools and plants, protecting and

maintaining the trench dry etc.,

stacking, cleaning the

excavated earth at a safe

distance out of the area

enclosed by the layout etc. all

complete and accepted by the

Engineer-in-charge, subject to

cum

28698.230

## Page 4

submit method statement of

carrying out excavation work to

the Engineer-in-charge for

approval.

However, engineer's

approval shall not relieve the

contractor of his responsibilities

and obligations under the

contract.

02.1.4: Added rate

for each additional 0.5 meter

depth exceeding 1.5 meter.(18)

9

1

16-620-

00

(BWDB)

Shoring for slope protection of

foundation trench, canal,

embankment, road, pond etc.

as per design slopes, grades

including removal of spoils to a

safe distance as per direction

of Engineer in charge. 16-620-

30: By local hard wood ballah

post of 6.0m length, 125mm

dia, 1.0m c/c, and 2.0m drive

with 6.0m long bamboo of

avarage 75mm dia, @ 1.0m c/c

and 2.0m drive with drum sheet

walling and average 70mm dia

half split bamboo batten @

2.0m c/c fixed with nails.

Sqm

1520.000

10

1

01.08

(LGED)

Bench Mark Pillar:

Manufacturing, supplying &

fixing in position RCC (1:2:4)

Bench Mark Pillars of size

150mm x 150mm x 750mm,

with 400mm x 400mm x

100mm base having 3 nos.

10mm dia MS bar each way at

base, 4 nos. 10mm dia vertical

bar and 8 nos. 6mm dia tie,

including cost of form works,

concreting, reinforcement,

plastering at top, inscribing on

exposed surface, finishing

surface, curing, earth cutting,

embedding 450mm below GL.,

backfilling, ramming etc.

complete as per direction of E-

I-C.

Nos

4

11

1

09.05

(PWD)

Providing and making point

welding at contact point of the

spiral binders at reasonable

intervals with the main

reinforcements by electric arc

welding for construction of cast

in situ bored pile carefully with

highly oxidized electrodes,

making the points prominent

and accepted by the Engineer-

in-charge (Rate is inclusive of

all materials labour, tools and

plants, electricity and all

equipment).

Point

100073.000

12

1

09.6

(PWD)

Providing and making welded

splice over two sides of

contact by welding of minimum

300 mm length at the lap of

main reinforcement in re-bar

cage to be placed in bore-hole

where necessary by electric

arc welding with highly oxidized

electrodes making the joint

prominent all complete and

accepted by the Engineer-in-

charge. (Rate is inclusive of all

materials labour, tools and

plants, electricity and all

equipment).

m of weld

24.000

13

1

4.07.01

BCIS: Boring for cast-in-situ

m

4243.500

## Page 5

(LGED)

piles up to the required depth

and specified diameter with

driving temporary steel casing

(shall be 8mm thick with

necessary stiffener bands and

sharp edge at bottom and

provided up to non-collapsible

strata from the existing ground

level but not less then 4.0 m) in

all types of soils including min.

2-chamber slurry tank, drilling

with bentonite circulation (Dry

Bentonite powder of liquid limit

of minimum 350 shall be mixed

with water @ minimum 4% by

weight to make the fresh

drilling fluid of viscosity

between 32-50 seconds and

density less than 1.1gm/cc),

maintaining fluid level inside

casing at all time at least 2 m

higher than outside the casings,

washing bore hole by air lift

cleanup method with fresh

bentonite slurry until the slurry

from bore hole bottom have

density less than 1.15gm/cc

and sand content is less than

4%, make the bore hole ready

for concreting including

disposal/removal of all bored

material, hire charge of

mechanical winch machine/ skid

mounted mechanical table drive

rotary/ hydraulic rotary boring

equipment, derrick, trimie pipe,

cost of fuel, lubricant,

mobilization, demobilization,

spares, insurance coverage,

water, electricity and other

charges all complete as per

design, drawing, specification

and direction of E-I-C.

Contractor shall submit the

methodology of cast-in-situ pile

work including information on

boring equipment, sequence of

boring & casting, quality

control, disposal of spoils, test

result of materials to the E-I-C

for approval before

commencing any boring

operation. Boring and

excavation for a pile shall not

commence until 24 hours after

completion of any pile within

radius of 6 meters c/c. Cost of

collecting, conveying, loading,

transportation of spoils/mud

accumulated during boring of

cast-in-situ pile with all lifts and

lead is included in this unit rate.

The spoils must be dumped in

an unobjectionable place

outside the site premises with

minimum traffic disruption and

the procuring entity will not be

responsible for any

irregularities by the party

regarding dumping of the

spoils. Note: Boring method

shall be selected as per

instruction given in the drawing.

4.07.01.01.04:

700mm

diameter

14

1

4.07.03

(LGED)

RCC-25SCCM: Reinforced

cement concrete work for cast-

Cum

1633.750

## Page 6

in-situ pile with minimum

cement content relates to mix

ratio 1:1.5:3 andmaximum

water cement ratio 0.4 having

minimum required average

compressive strength, f'cr =

33.5 Mpa and satisfying a

compressive strength f'c = 25

Mpa at 28 days on standard

cylinders as per standard

practice of code AASHTO/

ASTM and cement conforming

to BDS EN 197-1 : 2003

CEM-II/A-L/M/V/W 42.5N,

high range water reducing and

retarding admixture of

complying type G under ASTM

C 494 (Doses of admixture to

be fixed by the mix design),

sand of minimum FM 2.5 and

20mm down well graded

crushed stone chips broken

from boulders (Preferably

stone chips from Madhyapara,

Dinajpur, LAA value not

exceeding 30) conforming to

ASTM C 33, including breaking

chips, screening through proper

sieves, placing & maintaining

re-bar cage in position, jointing

longitudinal bars by welding or

re-bar coupling method, placing

and removing tripod/ derrick as

per requirement, mixing in

standard mixture machine with

hoper, maintaining allowable

slump of 150mm to 200mm,

placing a sliding plug or barrier

to prevent direct contact

between the first charge of

concrete in the pipe of the

tremie and the water of drilling

fluid, pouring the concrete in

bore hole with the help of trimie

pipe, maintaining the trimie pipe

immersed in concrete by at

least 1.5m throughout the

period of concreting etc.

including cost of all materials,

labour, equipment and all

incidental charges but excluding

the cost of reinforcement and

its fabrication, welding,

coupling and placing etc. all

complete as per design,

drawing, specifications and

direction of the E-I-C. The

contractor shall maintain a

continuous record of the

volume of concrete used and

the level of the concrete in the

pipe. Any deviations from the

theoretical, or expected,

volume/ level relationship shall

be immediately reported to the

E-I-C. Additional quantity of

cement to be added if required

to attain the specified strength

to be provided by the

contractor at his own cost.

Note: Using Concrete Mixer

15

1

09.7

(PWD)

Labour for breaking head of

hardened cast in situ bored

pile/ pre-cast pile up to a

required length by any means

but without damaging the rest

and removing the dismantled

Cum

42.620

## Page 7

materials such as concrete to a

safe distance including scraps

and cleaning concrete from

steel/ M.S. rods, straightening

and bending of pile bars,

preparation and making

platform where necessary,

carrying, all sorts of handling,

stacking the same properly

after clearing, leveling and

dressing the situ

16

1

4.07.23

(LGED)

SLT: Conducting static load

test as per ASTM D 1143 or

equivalent standard for the cast

in situ/ pre-cast pile providing

required land development

(earth exavation, sand filling,

sub-base, cement concrete

work, bullah palisading etc.),

Crib wall (sand bags, brick

works, CC blocks etc.)

depending on test loading,

scaffolding, bracing, jacks,

pressure test gauge, loading

unloading, arranging other

necessary plants and

equipment including staging,

mobilization, demobilization,

cost of empty gunny/ fertilizer/

plastic bag, concrete block/

steel sheet, sand and filling

sacs/ empty

gunny/fertilizer/plastic bags,

switching, loading and

unloading, record readings and

preparation of results in

standard forms and other

incidental charges as per

standard practice and

procedures including

submission of load test report,

furnishing all graph and chart

etc. complete in all respects

approved and accepted by the

E-I-C. Before commencing

load test, Contractor shall

submit methodology along with

combined calibration report for

conducting load test to the E-I-

C for approval. The Testing

firm will be selected taking

prior approval of procuring

entity. The methodology of

static load test and driving logs/

boring & pouring logs shall be

the part of test report. Load

Test and Report shall be

conducted under the

supervision of a professional

Geotechnical Engineer

registered in the Bangladesh

Professional Engineers

Registration Board (BPERB),

Institute of Engineers

Bangladesh (IEB). [Cost of

Combined Calibration Test

(Hydraulic Jack, pressure

gauge & electric/hydraulic

pump) has been included in all

sub-items.] 4.07.23.02: Above

75 MT and up to 125 MT

each

3.000

17

1

44-240-

00

(BWDB)

Supplying at site U-Type hot

rolled steel sheet pile of

different section of

Phosphorus=0.04%(Maximum),

Sulphur = 0.04% (Maximum),

M. ton

40.610

## Page 8

Tensile strength=

> 490 N/mm2

, Yield strength =

>295 N/mm2,

Elongation =17% (Minimum)

including all taxes, freights,

incidental charges etc.

complete as per as per detail

technical specification and

direction of the Engineer -in-

charge. 44-240-10: U-Type,

hot-rolled steel sheet pile

width= 400mm to 600mm:

height=

> 100mm, Thickness

>=10.5mm. (

�

0.50mm)

18

1

44-290-

00

(BWDB)

Driving steel sheet piles of

various sections and weights in

any type of soil, by mechanical

device including handling and

placing in position including

supply of all materials and

equipments related with driving

of sheet piles including

correcting leaning beyond

tolerance and other defects

etc. complete (measurement

will be taken on projected width

x height) as per direction of

Engineer in charge. 44-290-30:

U-Type or any other type :

Upto 6.50 m depth.

Sqm

317.250

19

1

44-320-

00

(BWDB)

Cutting of steel sheet piles to

design length and shape as per

requirement in design and

drawing and as per direction of

Engineer in charge. 44-320-20:

Above 10mm thick.

m

28.400

20

1

44-340-

00

(BWDB)

Fabrication and construction of

corner Joints in steel sheet

piles including cutting, welding,

shaping as per design and

specification and as per

requirement including supply of

necessary welding materials,

equipments and other

accessories as per

specification and direction of

Engineer in charge.

m

24.000

21

1

76-180-

00

(BWDB)

Cleaning corrosion and rust

from the surface of steel with

power brush etc. Complete

including the cost of all

materials as per direction of

Engineer in charge.

Sqm

2250.360

22

1

72-180-

00

(BWDB)

Provide 1 (one) coat of Zinc

phosphate as primary coat and

2 (two) coat of coaltar epoxy

coat over primary coat to steel

surface with paint of approved

colour etc. complete including

the cost of all materials as per

direction of Engineer in charge.

Sqm

2250.360

23

1

44-310-

00

(BWDB)

Supplying and placing 20mm

thick hessian cloth impregnated

with bitumen in expansion joints

or on top of sheet piles as per

specification and direction of

Engineer in charge.

Sqm

34.280

24

1

20-495-

00 (E.R)

BWDB)

Polythene Sheet : (1.0 kg per

6.50 sqm).

Sqm

536.700

25

1

12-310-

00

(BWDB)

Bailing out of water with all

leads and lifts by manual labour

or pump, with all arrangements

for protection of ring bund and

side slopes of foundation pit

Cum

164905.650

## Page 9

side slopes of foundation pit

against erosion or washout etc.

complete actual volume of

work will be measured by

sounding method before

starting the work) as per

direction of Engineer in charge.

12-310-20: By pump.

26

1

4.06.03

(LGED)

PCC-10: Plain cement

concrete work in foundation

with minimum compressive

strength of 10 MPa at 28 days

(sugessted mix proportion

1:3:6) on standard cylinder as

per standard practice of Code

AASHTO/ ASTM and cement

conforming to BDS EN 197-1 :

2003 CEM-II/A-L/M/V/W

42.5N, sand of minimum FM

1.8 and 20mm down well

graded 1st class/ picked brick

chips (LAA value not exceeding

40) comforming to ASTM C 33

including breaking bricks into

chips, shuttering, mixing by

concrete mixer machine,

casting, laying compacting and

curing for the requisite period

etc. all complete as per

direction of the E-I-C.

Additional quantity of cement to

be added if required to attain

the strength at the contractor's

own cost.

Cum

45.390

27

1

04.09.03

(LGED)

RCC-25SCCM: Reinforced

cement concrete work with

minimum cement content

relates to mix ratio 1:1.5:3 and

maximum water cement ratio

0.4 having minimum required

average strength, f'cr = 33.5

MPa and satisfying a

compressive strength f'c = 25

MPa at 28 days

on standard

cylinders as per standard

practice of Code AASHTO/

ASTM and cement conforming

to BDS EN 197-1 : 2003 CEM

II/A-L/M/V/W 42.5N, high

range water reducing admixture

of complying type A or F under

ASTM C 494 (Doses of

admixture to be fixed by the

mix design), sand of minimum

FM 2.5 and 20mm down well

graded crushed stone chips

broken from boulders

(Preferably stone chips from

Madhyapara, Dinajpur, LAA

value not exceeding 30)

conforming to ASTM C33

including breaking chips,

screening through proper

sieves, cleaning, placing shutter

in position, making shutter

water-tight properly, placing

reinforcement in position,

mixing in standard mixture

machine with hoper, maintaining

allowable slump of 75mm to

100mm, casting in forms,

compacting by mechanical

vibrator machine, curing for 28

days, removing centering-

shuttering after approved

specified time period,

other

incidental charges, etc. all

cum

461.260

## Page 10

complete as per drawing,

specification & direction of the

E-I-C. The cost of

reinforcement and it's

fabrication, welding, coupling,

placing, binding etc. is not

included but the cost of

admixture is included in this unit

rate. Additional quantity of

cement to be added if required

to attain the strength at the

contractor's own cost. [Using

Concrete Mixture Machine and

retail rate of Cement]

04.09.03.01: For pile caps,

abutment base,

facing

elements of Reinforced/

Mechanically Stabilized Earth

Structure, bottom slab of Box

Culvert etc.

28

1

04.09.03

(LGED)

RCC-25SCCM: Reinforced

cement concrete work with

minimum cement content

relates to mix ratio 1:1.5:3 and

maximum water cement ratio

0.4 having minimum required

average strength, f'cr = 33.5

MPa and satisfying a

compressive strength f'c = 25

MPa at 28 days

on standard

cylinders as per standard

practice of Code AASHTO/

ASTM and cement conforming

to BDS EN 197-1 : 2003 CEM

II/A-L/M/V/W 42.5N, high

range water reducing admixture

of complying type A or F under

ASTM C 494 (Doses of

admixture to be fixed by the

mix design), sand of minimum

FM 2.5 and 20mm down well

graded crushed stone chips

broken from boulders

(Preferably stone chips from

Madhyapara, Dinajpur, LAA

value not exceeding 30)

conforming to ASTM C33

including breaking chips,

screening through proper

sieves, cleaning, placing shutter

in position, making shutter

water-tight properly, placing

reinforcement in position,

mixing in standard mixture

machine with hoper, maintaining

allowable slump of 75mm to

100mm, casting in forms,

compacting by mechanical

vibrator machine, curing for 28

days, removing centering-

shuttering after approved

specified time period,

other

incidental charges, etc. all

complete as per drawing,

specification & direction of the

E-I-C. The cost of

reinforcement and it's

fabrication, welding, coupling,

placing, binding etc. is not

included but the cost of

admixture is included in this unit

rate. Additional quantity of

cement to be added if required

to attain the strength at the

contractor's own cost. [Using

Concrete Mixture Machine and

retail rate of Cement]

cum

416.060

## Page 11

04.09.03.02.01: For diaphragm

walls, wing walls, piers,

columns, projected pile cap

above water level,

pier caps,

abutments of bridges and

vertical members of box

culverts

For height up to 5m

29

1

04.09.03

(LGED)

RCC-25SCCM: Reinforced

cement concrete work with

minimum cement content

relates to mix ratio 1:1.5:3 and

maximum water cement ratio

0.4 having minimum required

average strength, f'cr = 33.5

MPa and satisfying a

compressive strength f'c = 25

MPa at 28 days

on standard

cylinders as per standard

practice of Code AASHTO/

ASTM and cement conforming

to BDS EN 197-1 : 2003 CEM

II/A-L/M/V/W 42.5N, high

range water reducing admixture

of complying type A or F under

ASTM C 494 (Doses of

admixture to be fixed by the

mix design), sand of minimum

FM 2.5 and 20mm down well

graded crushed stone chips

broken from boulders

(Preferably stone chips from

Madhyapara, Dinajpur, LAA

value not exceeding 30)

conforming to ASTM C33

including breaking chips,

screening through proper

sieves, cleaning, placing shutter

in position, making shutter

water-tight properly, placing

reinforcement in position,

mixing in standard mixture

machine with hoper, maintaining

allowable slump of 75mm to

100mm, casting in forms,

compacting by mechanical

vibrator machine, curing for 28

days, removing centering-

shuttering after approved

specified time period,

other

incidental charges, etc. all

complete as per drawing,

specification & direction of the

E-I-C. The cost of

reinforcement and it's

fabrication, welding, coupling,

placing, binding etc. is not

included but the cost of

admixture is included in this unit

rate. Additional quantity of

cement to be added if required

to attain the strength at the

contractor's own cost. [Using

Concrete Mixture Machine and

retail rate of Cement]

04.09.03.02.02: For diaphragm

walls, wing walls, piers,

columns, projected pile cap

above water level,

pier caps,

abutments of bridges and

vertical members of box

culverts

For height above 5m to

10m

cum

82.790

30

1

04.09.03

(LGED)

RCC-25SCCM: Reinforced

cement concrete work with

minimum cement content

relates to mix ratio 1:1.5:3 and

cum

38.870

## Page 12

maximum water cement ratio

0.4 having minimum required

average strength, f'cr = 33.5

MPa and satisfying a

compressive strength f'c = 25

MPa at 28 days

on standard

cylinders as per standard

practice of Code AASHTO/

ASTM and cement conforming

to BDS EN 197-1 : 2003 CEM

II/A-L/M/V/W 42.5N, high

range water reducing admixture

of complying type A or F under

ASTM C 494 (Doses of

admixture to be fixed by the

mix design), sand of minimum

FM 2.5 and 20mm down well

graded crushed stone chips

broken from boulders

(Preferably stone chips from

Madhyapara, Dinajpur, LAA

value not exceeding 30)

conforming to ASTM C33

including breaking chips,

screening through proper

sieves, cleaning, placing shutter

in position, making shutter

water-tight properly, placing

reinforcement in position,

mixing in standard mixture

machine with hoper, maintaining

allowable slump of 75mm to

100mm, casting in forms,

compacting by mechanical

vibrator machine, curing for 28

days, removing centering-

shuttering after approved

specified time period,

other

incidental charges, etc. all

complete as per drawing,

specification & direction of the

E-I-C. The cost of

reinforcement and it's

fabrication, welding, coupling,

placing, binding etc. is not

included but the cost of

admixture is included in this unit

rate. Additional quantity of

cement to be added if required

to attain the strength at the

contractor's own cost. [Using

Concrete Mixture Machine and

retail rate of Cement]

04.09.03.03.02: For solid slab

type super-structure including

cantilever, side walk, curb and

wheel guard of bridges

For

height above 5m to 10m

31

1

08.1

(PWD)

Supplying, fabrication and fixing

to detail as per design : ribbed

or deformed bar reinforcement

(excluding laboratory test fees)

for Reinforced concrete,

produced and marked in

accordance with BDS ISO

6935-2:2016 (or standard

subsequently released from

BSTI) including straightening

and cleaning rust, if any,

bending and binding in position

with supply of G.I. wires,

conducting necessary

laboratory tests etc. (excluding

splices or laps) complete in all

respect and accepted by the

Engineer-in-charge

(Measurement shall be

Kg

227582.810

## Page 13

recorded only on standard

mass per unit length of bars,

while dia of bars exceeds its

standard)

08.1.2: Grade 400

(B400DWR / B420DWR:

complying BDS ISO 6935-

2:2016 / ASTM A615) ribbed

or deformed bar produced and

marked according to

Bangladesh standard, with

minimum yield strength, fy

(ReH)= 400 MPa but fy not

exceeding 480 MPa and

whatever is the actual yield

strength within allowable limit

as per BNBC/ ACI 318, the

ratio of ultimate tensile strength

fu to yield strength

fy, shall be

at least 1.25 and

minimum

elongation after fracture and

minimum total elongation at

maximum

force is 17% and 8%

respectively : up to ground

floor.

32

1

26.42

(PWD)

Supplying, fitting and fixing of

G.I. pipe with all special

fittings, such as bends, elbows,

sockets, reducing sockets,

Tee, unions, jam-nuts etc.

including cutting trenches

where necessary and fitting the

same with earth duly rammed

and fixing in walls with holder

bats and making hole in floors,

walls and consequent mending

good the damages etc. all

complete in all respects

approved and accepted by the

Engineer- in- charge. 26.42.5:

40 mm dia G.I. pipe with wall

thickness 3.25 mm, outside

diameter min 46.5 mm, weight

3.83 kg/m, can withstand min

50 kg/cm2 hydraulic pressure.

m

87.240

33

1

76-630-

00

(BWDB)

Supply and fitting and fixing

23cm wide P.V.C water stops

having minimum strength of

13.80N/mm2 at 225%

elongation and of approved

quality in antraction and

expansion joints with necessary

arrangements for modification

in shuttering and keeping the

water stop in position etc.

complete as per design,

specification anddirection of

Engineer in charge.

76-630-10:

3 bulb type.

m

45.000

34

1

76-170-

00

(BWDB)

M.S. Work in plates, angles,

channels, flat bars, Tees etc.

including fabricating, machining,

cutting, bending, welding,

forging, drilling, revetting,

embedding anchor bars,

staging and fitting, fixing, local

handling etc. comlpete with

energy consumption and supply

of labours including the cost of

materials as per design,

specification and direction of

Engineer in charge.

Kg

26695.940

35

1

72-130-

00

(BWDB)

Synthentic enamel painting to

steel surfaces/ gates/ hoists

etc. with paint of approved

colour (2 coats over a coat of

Sqm

50.100

## Page 14

priming) including preparation

of the surface with steel brush,

emery paper and cleaning etc.

complete including the cost of

all materials as per direction of

Engineer in charge. 72-130-10:

2 coats over a priming coat.

36

1

04-280-

00

(BWDB)

Constructing at site, cement

mortar gauge on masonry wall,

including engraving in meter,

decimeter & centimeter,

painting and figuring with black

and red water proof paint, etc.

complete as per direction of

Engineer in charge. 04-280-10:

150mm x 25mm

m

11.200

37

1

16-560-

00

(BWDB)

Back filling in hydraulic

structures including all leads

and lifts with sand in 150mm

layer including watering,

ramming, compacting to 30%

relative density etc. complete

by compactor or any other

suitable method as per

direction of Engineer in charge.

16-560-20: Sand of FM

>=0.80

Cum

4968.210

38

1

16-550-

00

(BWDB)

Back filling in hydraulic

structures and slope building in

protective works including all

leads and lifts with selected

local soil in layer of 150mm

including watering, ramming

etc. complete compacted to

20% relative density by

compactor or any other

suitable method as per

direction of Engineer in charge.

Cum

9096.470

39

1

40-520-

00

(BWDB)

Supplying and laying dry 1st

class or pick jhama chips as

filter in two layers (top and

bottom) as per specific size,

range and gradation, including

breaking chips, grading,

preparation of surface,

compacting each layer etc.

complete with supply of all

materials and as per direction

of Engineer in charge: 40-520-

20: Well graded between

40mm to 20mm size.

Cum

105.300

40

1

40-520-

00

(BWDB)

Supplying and laying dry 1st

class or pick jhama chips as

filter in two layers (top and

bottom) as per specific size,

range and gradation, including

breaking chips, grading,

preparation of surface,

compacting each layer etc.

complete with supply of all

materials and as per direction

of Engineer in charge: 40-520-

30: Well graded between

20mm to 5mm size.

(Combination of sub-item 10 &

30 or 20 & 30 shall be used)

Cum

105.300

41

1

40-550-

00

(BWDB)

Supplying and laying sand as

filter layers as per specific size

ranges and gradation including

preparation of surface,

compacting in layer etc.

complete with supply of all

materials and as per direction

of Engineer in charge. 40-550-

Cum

108.270

## Page 15

20: FM : 1.5 to 2.0

42

1

40-500-

00

(BWDB)

Supplying and placing non-

woven needle punched type

geotextile fabric (97%

Polypropylene Fabric and

additives shall be 3%, Carbon

Black 2% and Antioxidants 1%,

may be mixed as additives, unit

weight : 855 Kg/m3 to 946

Kg/m3) as filter materials of

elongation at maximum force

machine direction (MD)

>=60%

and <

= 100 % , elongation at

maximum force (CMD) =

>

40% and <

= 100% ,horizontal

and vertical permeability (under

2 kn/m

�

pressure)=

>2x10E-3

m/sec. for effective erosion

protection in hydraulic

structures/river training works

including local handling, placing

in position, providing machine

seamed joints (with 100%

polypropylene or nylon thread)

or 35cm lap in dry condition or

minimum 100cm lap under

water including protecting the

geotextile material from UV ray

and from any other damages

including supply of all materials,

labours, equipment's etc.

complete as per direction of

Engineer in charge. (Geotextile

delivered at site should be

certified by ISO and

clearlylabelled with brand name

and grade printed at regular

intervals across the body of the

fabric). 40-500-40: Mass =

>400 gm/m

�

, thickness(Under

2 kpa pressure) =

>3.00 mm,

EoS<

=0.08mm, strip tensile

strength =

>23 kn/m, grab

strength

=

>1500 N, CBR

puncture resistance =

>3800 N

Sqm

1169.950

43

1

40-230-

00

(BWDB)

Manufacturing and supplying

C.C. blocks in leanest mix.

1:2:4 with cement, sand (FM

>=2.0) and Stone Chips

(40mm down graded) to attain

a 28 days cylinder strength of

18 N/mm

�

including grading,

washing stone chips, mixing,

laying in forms, consolidation,

curing for at least 21 days,

including preparation of

platform, shuttering and

stacking in measurable stacks

etc. complete including supply

of all materials (steel shutter to

be used) as per direction of

Engineer in charge. 40-230-90:

Block Size: 50cmx50cmx20cm.

Each

1118

44

1

40-230-

00

(BWDB)

Manufacturing and supplying

C.C. blocks in leanest mix.

1:2:4 with cement, sand (FM

>=2.0) and Stone Chips

(40mm down graded) to attain

a 28 days cylinder strength of

18 N/mm

�

including grading,

washing stone chips, mixing,

laying in forms, consolidation,

curing for at least 21 days,

including preparation of

platform, shuttering and

stacking in measurable stacks

Each

1099

## Page 16

etc. complete including supply

of all materials (steel shutter to

be used) as per direction of

Engineer in charge. 40-230-45:

Block Size: 35cmx35cmx35cm.

45

1

40-230-

00

(BWDB)

Manufacturing and supplying

C.C. blocks in leanest mix.

1:2:4 with cement, sand (FM

>=2.0) and Stone Chips

(40mm down graded) to attain

a 28 days cylinder strength of

18 N/mm

�

including grading,

washing stone chips, mixing,

laying in forms, consolidation,

curing for at least 21 days,

including preparation of

platform, shuttering and

stacking in measurable stacks

etc. complete including supply

of all materials (steel shutter to

be used) as per direction of

Engineer in charge.

40-230-30:

Block Size: 45cmx45cmx30cm

Each

4301

46

1

40-270-

00

(BWDB)

Labour charge for protective

works in laying CC blocks of

different sizes including

preparation of base, watering

and ramming of base etc.

complete as per direction of

Engineer in charge. 40-270-20:

200 m to 500 m.

Cum

364.310

47

1

76-242-

00

(BWDB)

Manufacturing & Supplying of

M.S. Vertical Lift Gate shutter

having 10mm thick M.S. skin

plate and stiffener with

minimum 75mm x 75mm x 9mm

M.S. angle (Top & Bottom

beam, Horizontal main beam

and End vertical beam) as

frame, ?28mm x 53mm x

15mm P-type top & side rubber

seal fixed with ?10mm x 55mm

S.S. counter shank bolts with

nuts and 25mm x 6mm S.S.

strip as clamp drilled spaces @

150mm c/c, Utype or Flat

bottom rubber seal, threaded

stem attachment with bottom

socket, nut-bolts, cotter pin and

washer as per approved design

including the cost of all

materials of proper grade &

brand new with a prime coat of

redoxide and three final coat of

coaltar epoxy paint (marine

quality) where necessary as

per specification and direction

of Engineer in charge. 76-242-

50: Size 1.65m x 1.875m (Vent

1.5m x 1.8m)

Each

3

48

1

76-252-

00

(BWDB)

Manufacturing & Supplying of

M.S. Flap Gate shutter of

10mm thick M.S. skin plate and

stiffener with minimum 75mm x

75mm x 9mm M.S. angle as

frame, horizontal & vertical

beam, ?28mm x 53mm x 15mm

P-type rubber seal, fixed with ?

10mm x 55mm S.S. counter

shank bolts with nuts and

25mm x 6mm S.S. strip as

clamp drilled spaces @ 150mm

c/c, hinge assy with gate and

wall bracket, link arm of (16-

20)mm thick M.S. plate, 4 nos

Each

3

## Page 17

?(35-50)mm S.S. hinge pin

with proper thread, nut-bolts,

cotter pin and washer as per

approved design including the

cost of all materials of proper

grade & brand new with a

prime coat of red-oxide and

three final coat of coaltar

epoxy paint (marine quality)

where necessary as per

specification and direction of

Engineer in charge. 76-252-50:

Size 1.65m x 1.95m (Vent

1.5m x 1.8m)

49

1

76-260-

00

(BWDB)

Labour charge for fitting and

fixing of M.S. vertical lift gate/

flap gate shutters of different

size including making holes in

concrete for hooking

arrangements with supply of

necessary materials, tools and

other accessories required for

fitting the same to

regulator/sluice and mending

the damages with CC (1:2:4),

removing the spoils etc.

complete including the cost of

all materials as per direction of

Engineer in charge. 76-260-20:

Size 1.95m x 1.35m or 1.95m x

1.65m.

Each

6

50

1

76-190-

00

(BWDB)

Manufacturing, supplying and

Installation of Padestal type

lifting device for slide gate with

63mm dia threaded steel shaft,

146mm outer dia bronze nut,

thrust bearing, steel bevel gear

etc. as per approved design

including supply of all

components, labours with a

prime coat of redoxide where

necessary etc. complete

including the cost of all

materials as per specification

and direction of Engineer in

charge.

Each

3

51

1

76-780-

00

(BWDB)

Supplying and fitting, fixing

dowel bar in expansion each

joints of hydraulic structures

with 32mm dia M.S. plain bar

2000mm long, hooked in one

end and painted at the other

end for a length of 900mm with

40mm dia 100mm long

G.I./M.S. pipe closed at one

end and filled with 30mm thick

compressible filter, including

placing in position, binding with

G.I. Wire etc. complete as per

direction of Engineer in charge.

Each

74.000

52

1

16-470-

00

(BWDB)

Earth work by manual labour, in

all kinds of soil in removing the

cross bundh/ ring bundh,

including all leads and lifts

complete and placing the spoils

to a safe distance, (minimun

15m apart from the bank) as

per direction of Engineer in

charge.

Cum

6503.640

53

1

16-240-

00

(BWDB)

Earth work by Mechanical

Excavator (Long Boom) in

constructing/ resectioning of

embankment/canal bank/ road

etc. compacted to 85%/90%

maximum dry density at

Cum

4234.330

## Page 18

optimum moisture content, with

reference to laboratory density

test AAHSTO modified

hammer, with clayey soil(minm

30% clay, 0 40% silt, 0-30%

sand) within the initial lead of

30m and all lifts including

throwing the spoils to profiles in

layers not exceeding 230mm in

thickness with clod breaking to

a maximum size of 100mm,

benching the side slopes,

removing roots and stumps of

trees of girth upto 200mm from

the ground, stripping/ploughing

the base of embankment and

borrow pit area, dug bailing,

rough dressing including

150mm cambering at the

centre of crest etc. complete,

including maintenance of the

same for 6 months after

completion, (compaction will be

done by the contractor with

approved equipment, including

all ancillary charges for

compaction and testing) as per

direction of Engineer in charge.

16-240-10: Embankment by

Mech. Equipment; ht: 0 to 4m;

85% comp

54

1

48-100-

00

(BWDB)

Fine dressing and close turfing

of the slopes and the crest of

embankment with 75mm thick,

good quality durba or

charkanta sods of size 200mm

x 200mm, with all leads and

lifts, including ramming,

watering until the turf grows

properly, maintaining etc.

complete (measurement will be

given on well grown grass

only). as per direction of

Engineer in charge.

Sqm

21440.000

55

1

16-410-

00

(BWDB)

Earth work by Mechanical

Excavator ( Long Boom ) in all

kinds of soil in excavation / re-

excavation of

channel/canal/khal etc.

Including disposal of spoil-soil

upto 30m away from the point

of excavation with rough

dressing and levelling etc.

complete as per direction of

Engineer-in charge.

Cum

22611.770

56

1

16-400-

00

(BWDB)

Earth work by manual labour in

all kinds of soil in excavation or

reexcavation of channels with

the initial lead of 30m and lift of

1.5m including levelling,

dressing and throwing the

spoils to profile with breaking

clods, rough dressing, clearing

jungles including cutting trees

upto 200mm girth, dug bailing

etc. complete as per direction

of Engineer in charge.

Cum

14115.160

57

1

16-420-

00

(BWDB)

Extra Rate for every additional

shift of Dredged/Excavated

earth or shifting of earth by

Mechanical Excavator ( Long

Boom ) of 30m or part thereof

beyond the initial shift of 30m

upto a maximum of 9 shift for

all kind of earthwork shift.

Cum

4234.550

## Page 19

(Unit: per shift per cum)

58

1

16-720-

00

(BWDB)

Filling ditch/pond/channel/khal

etc or land development/

improvment by dredged earth

from river bed (all kind of soil

but excluding organic material)

, carried Bulkhead/cargo/boat

or any other mean, loading and

unloading/Disposing/placing the

dredged materials in the

designated area upto 1 km

from river bank by bulkhead

dredger including , maintaining

slopes, levelling and dressing in

layers upto finished level with

all lifts & leads etc. all complete

as per direction of the Engineer

in charge.

16-720-10:

Bulkhead/cargo/boat or any

other

means within 1 km along

the river

Cum/ Km

5522.800

59

1

16-720-

00

(BWDB)

Filling ditch/pond/channel/khal

etc or land development/

improvment by dredged earth

from river bed (all kind of soil

but excluding organic material)

, carried by

Bulkhead/cargo/boat or any

other mean, loading and

unloading/Disposing/placing the

dredged materials in the

designated area upto 1 km

from river bank by bulkhead

dredger including, maintaining

slopes, levelling and dressing in

layers upto finished level with

all lifts & leads etc. all complete

as per direction of the Engineer

in charge. 16-720-20:

Bulkhead/cargo/boat or any

other means: 2nd km and

above.

Cum/ Km

5522.800

60

1

16-100-

00

(BWDB)

Erection of bamboo profile with

full bamboo posts and pegs not

less than 60mm in diameter

and coir strings etc. complete

as per direction of Engineer in

charge.

nos

22

61

1

10-200-

00

(BWDB)

Topography Survey of

embankment/road/canal/river

Section (Section with upto

50m), including cost of all

necessary equipment (Level

machine, Hand GPS, DGPS

etc.), Necessary Personnel

(River Morphology Expert for

data consistency checking,

Engineer/Survey specialist for

data processing with survey

software and prepare

necessary table, Graph and

Morphological Parameter

Calculations, Surveyor &

Unskilled labour for field data

collection), Accommodation,

Transportation from

Headquarter to site and site

movement and shaded boat for

bathymetric survey for field

survey Team including

necessary fuels and lubricant.

Other necessary auxillary items

such as survey flags, pegs with

provision for submission of

report in hard binded copies (at

nos

28

## Page 20

least 6 copies required), soft

copies and raw survey data

with provision of 10% random

re-survey for data consistency

checking as per direction of

Engineer Charge. 10-200-30:

Section interval: 101m to 150

m

62

1

10-250-

00

(BWDB)

Level Flying for survey work

Including cost of all necessary

equipments (Level Machine,

Hand GPS, DGPS etc.),

Necessary Personnel (River

Morphology Expert for data

consistency checking,

Engineer/Survey Specialist for

data consistency checking,

Engineer/Survey Specialist for

data processing with survey

software and prepare

necessary Table, Graph and

Morphological Parameter

Calculations, Surveyor &

Unskilled Labour for field data

collection), Accormmodation,

Transportation from

Headquarter to site and site

movement and shaded boat for

bathymetric survey for field

Survey Team including

necessary fules and Lubricant,

other necessary auxiliary items

with provision for submission of

report in hard binded copies(at

least 6 copies required), soft

copies and raw survey data

Consistency checking as per

direction of

Engineer in

Charge.

nos

4.200

63

1

32.6.1.1

(PWD)

Dismantling of R.C.C.

beam,

lintel, column, drop wall, sun

shade from any height

and

removal of debris to a safe

distance.

Cum

126.000

64

1

01.1.6

(PWD)

Providing and maintenance one

project profile signboards to be

placed at a suitable place of

the site including submission of

proposals for the materials of

the signboards and text layout

containing 3D picture, safety

instructions, project information

with security light etc. to the

Engineer-in-charge for approval

which will be positioned as

directed by the Engineer-in-

charge and removing the same

on completion of the works or

as instructed by the Engineer-

in-charge.

Sqm

3.750

65

1

01.7

(PWD)

Mobilization and cleaning site

before commencing actual

physical work and during

contract period and

demobilization after completion

of the works under contract to

be accepted by the Engineer-

in-charge. This work shall also

cover cleaning and clearing,

cutting or filling, dressing the

project area on and in the

ground to an extent that all the

events of works of the project

can be executed smoothly in a

working environment with a

Sqm

3000.000

## Page 21

particular attention on safety

and security in all respects, and

to stockpile the end outcome to

a place for disposal agreed by

the Engineer-in-charge, where,

payments are to be based on

ground area determined by the

Engineer-in-charge and be

proportionate to the

percentage progress of work

under contract as a whole in all

respects and approved by the

Engineer-in-charge.

66

1

12-110-

00

(BWDB)

Mobilization of complete well

point system, equipments and

accessories from one place to

another including loading and

unloading as per direction of

Engineer in charge. 12-110-10:

Within 1.00 km.

ps/ km

1

67

1

12-120-

00

(BWDB)

Installation of well point

dewatering set including supply

of all accessories, materials

and labours for one 5 cusec

vacuum pump for minimum 100

well points in controlling

subsurface water during

construction, providing

shrouding materials, conducting

test operation and subsequent

removal of the complete

dewatering set in accordance

with standard specification and

field requirement, as per

direction of Engineer in charge.

set

1

68

1

12-150-

00

(BWDB)

Operation of well point system,

by 5 cusec vacuum pump for

dewatering of subsurface

water, maintaining draw down

records including supply of

necessary fuels, lubricants,

operation personnel and

maintenance of the same upto

completion of the work in

accordance with standard

specification and direction of

Engineer in charge.

hours

1200.000

69

1

12-180-

00

(BWDB)

Installation of dewatering

tubewells at site for controlling

subsurface water during

construction, including supply of

all materials, equipments, fuels,

lubricants etc. providing

shrouding materials, conducting

well testing &development and

subsequent removal of

complete dewatering system in

accordance with standard

specification and field

requirement etc. complete

including sealing of hole,

mobilization and demobilization

of equipments and accessories

(contractor has to prepare and

submit a dewatering plan

based on field requirement to

the competent authority for

approval, prior to installation of

the tube well) as per direction

of Engineer in charge. 12-180-

10: 18.0m depth including

strainer (as per design) and

75mm dia tubewell (bore hole

200mm dia) with PVC pipe and

strainer.

No.

1

## Page 22

70

1

12-200-

00

(BWDB)

Operation of tubewell by

centrifugal pump for dewatering

of subsurface water,

maintaining drawdown records

including supply of necessary

fuels, lubricants, operation

personnel and maintenance of

the same upto the completion

of the work as per specification

and direction of Engineer in

charge. 12-200-20: 0.50 cusec

capacity

hours

400.000

71

1

12-160-

00

(BWDB)

Demobilization of complete well

point system, equipments and

accessories from one place to

another including loading and

unloading as per direction of

Engineer in charge. 12-160-10:

Within 1.00 km.

ps/ km

1

Grand

Total :

Schedule of Day works

Table

Name :

.

Item

no.

Group

Item

Code

(if

any)

Description

of Item

Measurement

Unit

Nominal

Quantity

Unit Price in Figures

(BDT)

Unit

Price

in

Words

(BDT)

Total

Price in

Figures

(BDT)

Total

Price

in

Words

(BDT)

N/A

1

N/A

Construction

of Regulator

(3-Vent:

1.50m x

1.80m)

N/A

1

Grand

Total :

Discount Form

Table

Name :

Unconditional Discount Form

Discount in Percentage (%)[In figure]

Discount in Percentage (%) [In word]

Provisional Sum

Instruction for Procuring Entity (PE):

This is an optional form.

PE may fill it if require. If it is filled then PE has to add this form to Grand Summary.

Table

Name :

Provisional Sum

Item

No

Description of

Specified

Provisional

Measurement

Unit

Quantity

Unit

Price In

figures

Unit

Price

In

Total

Price In

figures

Total Price

In

Tenderer

Response

## Page 23

No

Provisional

Sums

Unit

figures

(BDT)

words

(BDT)

figures

(BDT)

words(BDT)

Response